Stopwatch Design Report

# 1. Overview

The stopwatch module is a synthesizable Verilog design intended for Vivado behavioral simulation. It counts elapsed time in minutes (0–59) and seconds (0–59) using a parameterized clock divider, enabling fast simulation without requiring real-time clock frequencies. The design is controlled by three inputs — clock (clk), synchronous reset (rst), and a start/pause signal (start) — and exposes two 6-bit outputs for seconds and minutes respectively.

# 2. Module Description

The top-level module stopwatch contains three always blocks, each handling a distinct responsibility. The CLK_DIV parameter (default 5 for simulation) controls how many clock cycles constitute one simulated second.

| Port | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1-bit | System clock |
| rst | Input | 1-bit | Synchronous active-high reset |
| start | Input | 1-bit | 1 = run, 0 = pause |
| sec | Output | 6-bit | Seconds count (0 to 59) |
| min | Output | 6-bit | Minutes count (0 to 59) |

# 3. Module Integration

The three always blocks inside the module are tightly integrated in a producer-consumer chain:

- Clock Divider Block: counts up to CLK_DIV−1 on every rising edge of clk. When it reaches the limit, it resets to zero and asserts the internal tick signal for exactly one clock cycle. If start = 0, the divider freezes and no tick is produced, effectively pausing the stopwatch without losing the current count.

- Seconds Counter Block: increments sec on every tick. When sec reaches 59, it wraps back to 0 and simultaneously asserts min_tick for one cycle, signaling the minutes counter that one full minute has elapsed.

- Minutes Counter Block: increments min only when min_tick is high. It wraps from 59 back to 0 autonomously. A synchronous reset (rst = 1) clears all three registers — div_cnt, sec, and min — simultaneously, ensuring a clean restart from zero.

# 4. Testbench Simulation Scenario

The testbench (stopwatch_tb.v) exercises the following sequence to validate all functional modes:

| Step | Action | Expected Behaviour |
| --- | --- | --- |
| 1 | Reset (20 cycles) | sec = 0, min = 0 |
| 2 | Start counting | sec increments every simulated second |
| 3 | Pause at 30 s | sec holds at 30, min stays 0 |
| 4 | Resume counting | sec continues from 30 |
| 5 | Reset at 1 min 10 s | sec = 0, min = 0 (both cleared) |
| 6 | Fresh start, count 1 minute | sec wraps 59 → 0, min becomes 1 |

# 5. Simulation Parameters

| Parameter | Value |
| --- | --- |
| Timescale | 1 ns / 1 ps |
| Clock period | 10 ns (100 MHz) |
| CLK_DIV | 5 → 1 second = 50 ns |
| 1 simulated minute | 3000 ns |